/**

\page tutorial-install-raspberry Tutorial: Installation from source on Raspberry Pi

\tableofcontents

In this tutorial you will learn how to install ViSP from source on <a href="http://www.raspberrypi.org/help/what-is-a-raspberry-pi">Raspberry Pi 1 or Pi 2</a> equipped with an optional HD camera module.

\image html img-raspberry-pi.jpg

In a first section we give some useful instructions to start with a Raspberry PI. Then in the second section, we focus on the installation of ViSP from source.

\note Concerning ViSP installation, we provide also other \ref tutorial.

\section install_raspberry Raspberry Pi installation

\subsection raspberry Setting up Raspberry Pi

There are a lot of documentation and tutorial that explain different ways to setup a Raspberry Pi. A good reference is the official page <a href="http://www.raspberrypi.org/">http://www.raspberrypi.org</a>

We suggest to start with NOOBS (New Out Of the Box Software). Bellow we resume the different steps that are fully described in <a href="http://www.raspberrypi.org/help/noobs-setup/">http://www.raspberrypi.org/help/noobs-setup</a>:

- Format you SD card (at least 4Go are requested) on your computer. To this end you can use "GParted partition editor"  under Ubuntu.

- Download Noobs "Offline and network install" zip file from <a href="http://www.raspberrypi.org/downloads/">http://www.raspberrypi.org/downloads</a>.

- Unzip all the files on the SD card.

- Insert your SD Card into your Raspberry Pi.

- Plug in your keyboard, mouse, monitor and Ethernet cables.

- Now plug in the USB power cable to your Pi.

- Your Raspberry Pi will boot, and a window will appear with a list of different operating systems that you can install. Select the "recommended Raspbian" checkbox and click on "Install" button.

- When the install process has completed, the Raspberry Pi configuration menu (raspi-config) will load. Here you are able to set the time and date for your region and enable a Raspberry Pi camera board, or even create users. You can exit this menu by using Tab on your keyboard to move to Finish.

\subsection login Logging in

The default login for Raspbian is username \c pi with the password \c raspberry.

\subsection update Update your Raspbian distribution

If your Raspberry Pi is connected to Ethernet you can update your Raspbian distribution:

\code

sudo apt-get update

sudo apt-get upgrade

\endcode

\subsection camera Setting up a Raspberry Pi camera

If you have a Raspberry Pi camera module see <a href="http://www.raspberrypi.org/help/camera-module-setup/">http://www.raspberrypi.org/help/camera-module-setup</a>. To resume, enable the camera using:

\code

sudo raspi-config

\endcode

Enter in menu 5/ to enable the camera.

Then load the camera module:

\code

sudo modprobe bcm2835-v4l2

\endcode

This will add the following modules:

\code

pi@raspberrypi ~ $ lsmod

Module Size Used by

bcm2835_v4l2 37611 0

videobuf2_core 30853 1 bcm2835_v4l2

v4l2_common 7792 1 bcm2835_v4l2

videodev 121362 3 bcm2835_v4l2,v4l2_common,videobuf2_core

\endcode

To check if the camera is recognized and connected, run:

\code

v4l2-ctl --list-formats

\endcode

After each Raspberry Pi reboot you need to relaunch the previous modprobe command. To load bcm2835-v4l2 module during reboot, you can edit \c /etc/modules file

\code

sudo nano /etc/modules

\endcode

and add a line with the name of the module:

\code

bcm2835-v4l2

\endcode

\subsection startx Start graphical user interface

To load the graphical user interface, type \c startx and press Enter on your keyboard. This will later allow to use ViSP vpDisplayX or vpDisplayOpenCV classes useful to display images in a X11 window.

\section install_visp ViSP installation

\subsection prerequisities Prerequisities

First you need to install the following packagages (g++, CMake, Subversion) that are requested to get and build ViSP:

\code

sudo apt-get install build-essential cmake-curses-gui subversion

\endcode

Then you can install the following packages (X11, lapack, gsl, v4l, xml, png, jpeg, opencv), that correspond to the optional 3rd party libraries that are interfaced with ViSP:

\code

sudo apt-get install libx11-dev liblapack-dev libgsl0-dev libv4l-dev libxml2-dev libpng12-dev libjpeg-dev libopencv-dev

\endcode

The complete list of 3rd party libraries is detailed following  <a href="http://team.inria.fr/lagadic/visp/libraries.html">http://team.inria.fr/lagadic/visp/libraries.html</a> link.

\subsection get_source Getting ViSP source code

There are different ways to get ViSP source code on Raspberry Pi:

- You can download the <a href="http://team.inria.fr/lagadic/visp/download.html#latest">latest stable release</a> tarball. Once downloaded, uncompress the file using:

\code

mkdir /home/pi/soft; cd /home/pi/soft

wget http://gforge.inria.fr/frs/download.php/latestfile/475/ViSP-2.10.0.tar.gz

tar xvzf ViSP-2.10.0.tar.gz

\endcode

- You can also download the cutting-edge version of the source code from Subversion repository using:

\code

mkdir /home/pi/soft; cd /home/pi/soft

svn checkout svn://scm.gforge.inria.fr/svn/visp/trunk/ViSP ViSP-code

\endcode

We suppose now that ViSP source is in a directory denoted  \<source_dir\>, for example \c /home/pi/soft/ViSP-2.10.0 or \c /home/pi/soft/ViSP-trunk

\subsection config Configuring ViSP from source

- Create first a directory denoted \<binary_dir\> where you want to build ViSP. This directory will contain generated Makefiles, object files, and output libraries and binaries.

\code

cd /home/pi/soft; mkdir ViSP-build-release

\endcode

- Enter the \<binary_dir\> and configure the build type:

\code

cmake [<optional parameters>] <source_dir>

\endcode

For example:

\code

cd /home/pi/soft/ViSP-build-release

cmake -DCMAKE_BUILD_TYPE=RELEASE -DBUILD_SHARED_LIBS=ON ../ViSP-2.10.0

\endcode

A more versatile way to configure the build is to use \c ccmake, the CMake GUI:

\code

ccmake ../ViSP-2.10.0

\endcode

The following image shows that this command allows to configure (just by pressing [c] key) the build in a more advanced way where some options could be easily turned On/Off. It allows also to see which are the 3rd parties that will be used. Then to generate the makefiles, just press [g] key in the ccmake gui.

\image html img-ccmake-raspberry-pi.png Snapshot of the ccmake \c ../ViSP-2.10.0 command used to configure ViSP.

- There is an other way to see which are the 3rd parties that are found during the configuration stage and that will be used by ViSP during the build. To this end you can have a look to the text file named ViSP-third-party.txt and located in \<binary_dir\>. We provide hereafter an example of a possible content of this file:

\code

ViSP third-party libraries

Below you will find the list of third party libraries used to

build ViSP on your computer.

Mathematics:

Gnu Scientific Library      : yes

Lapack/blas                 : yes

Simulator:

Ogre simulator              : no

\- Ogre3D                   : no

\- OIS                      : no

Coin simulator              :

\- Coin3D                   : no

\- SoWin                    : no

\- SoXt                     : no

\- SoQt                     : no

\- Qt4                      : no

\- Qt3                      : no

Robots

Afma6                       : no

Afma4                       : no

Biclops                     : no

Ptu46                       : no

Pioneer                     : no

Viper S650                  : no

Viper S850                  : no

Video devices (display)

X11                         : yes

GTK                         : no

OpenCV                      : yes

GDI                         : no

Direct3D                    : no

Framegrabbers

Firewire libdc1394-2.x      : no

Video For Linux Two         : yes

DirectShow                  : no

CMU 1394 Digital Camera SDK : no

OpenCV                      : yes

Specific devices

Yarp                        : no

Kinect                      : no

\-libfreenect               : no

\-libusb-1.0                : no

\-pthread                   : yes

Video and image Read/Write:

FFMPEG                      : no

libjpeg                     : yes

libpng                      : yes

Misc:

XML2                        : yes

pthread                     : yes

OpenMP                      : yes

zbar                        : no

dmtx                        : no

Documentation:

Doxygen                     : no

Graphviz dot                : no

ViSP built with C++11 features: no

\endcode

\subsection build Building ViSP from source

- To build ViSP proceed with:

\code

make

\endcode

- To install ViSP proceed with:

\code

sudo make install

\endcode

- To build ViSP documentation, you have first to install Doxygen package:

\code

sudo apt-get install doxygen graphviz texlive-latex-base

\endcode

Then you can proceed with:

\code

make visp_doc

\endcode

You are now ready to see the next \ref tutorial-getting-started that will show you how to use ViSP as a 3rd party to build your own project on Raspberry Pi or on any other system. Then if you have a Raspberry Pi camera module, you can also follow the \ref tutorial-tracking-blob especially subsection Tracking form v4l2 live cameras.

*/